To the R&TTE Notified Body

We,

| Company name |  |
| --- | --- |
| Address |  |
| City |  |
| Country |  |

declare that our equipment identified by

| Product Description |  |
| --- | --- |
| Tradename or Brand(s) |  |
| Type or Model(s) |  |

as submitted for this Notified Body assessment, fulfils the requirements of Directive 99/5/EC and RF exposure limits specified  in Council Recommendation 1999/519/EC. Justification is given in the calculation following below:

| The measured conducted output power = Pconducted = a = |  | mW |
| --- | --- | --- |
| The maximum possible duty cycle (use a conservative value) = Pmax, % = b = |  | % |
| Paverage, conducted = a * b / 100 = c = |  | MW (calculated) |
| Paverage, conducted = 10 * log10 (c) = d = |  | dBm (calculated) |
| Maximum antenna gain = Gmax = e = |  | dBi |
| Maximum antenna cable Loss = Lmax = f = |  | dB |
| Paverage, eirp = d + e – f = g = |  | dBm (calculated) |
| Paverage, eirp = 10 (g / 10) = h = |  | mW (calculated) < 20mW ? |

(choose either A or B. In case of B, finish the calculation under B)

|  | A. This equipment complies with the 20mW limit and is deemed to be safe under all circumstances. |
| --- | --- |
| As calculated above, the average eirp (or eirp) appears to be less than 20mW and therefore complies with EN50371. |  |

|  | B. This equipment is a mobile (fixed) device and complies with the MPE limit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| According to EN50385 and EN50383, the far field calculation can be used for base stations (non hand held devices) and it shows that at a certain distance, the power density equals the MPE limit as specified for the target user group. This distance, preferably with some extra margin, shall always be respected when the equipment is transmitting. |  |  |  |  |  |  |  |
| For the |  |  | general public or |  | occupational workers (cross either general public or occupational workers), |  |  |
| the worst case MPE given as regulatory limit = MPE = i = |  |  |  |  |  |  | mW/cm2 |
| at frequency (this frequency must be within the equipment's frequency range) = j = |  |  |  |  |  |  | MHz |
| The minimum distance to maintain to the human body (Friis) = ( { (i / h)2 } / 4π )-1 = |  |  |  |  |  |  | cm (safe distance) |

Attestation:

| City and Country: | Date: | Name: (this must be a person) | Function: | Signature: (or official company stamp) |
| --- | --- | --- | --- | --- |